College Network Project

Connecting the Data Science and Computer Science Departments

Prepared by: Aseel Alanzi

Computer Science Department, University of Hail

Tool Used: Cisco Packet Tracer

Date: November 2025

1. Objective

This project designs and configures two interconnected departmental networks within the College of Computer Science at the University of Hail — one for the Data Science Department and another for the Computer Science Department — using Cisco Packet Tracer. It demonstrates core networking concepts including IP addressing, DNS configuration, static routing, and inter-network communication through routers.

2. Project Requirements

- Create two separate Class C networks, one per department.

- Connect both networks using two routers linked via a Serial connection.

- Assign IP addresses to all router interfaces (FastEthernet + Serial), servers, and PCs.

- Configure a DNS server in each network to resolve device names to IP addresses.

- Test connectivity using ping between two devices in the same network, by device name.

- Test connectivity using ping between two devices in different networks, by IP address.

- Add clear labels and organize the topology for readability.

3. Network Design Overview

The network is composed of two departmental sub-networks connected through a dedicated router-to-router serial link:

- Network A — Data Science Department: 192.168.10.0/24

- Network B — Computer Science Department: 192.168.20.0/24

- Router Serial Link: 10.10.10.0/30

Total devices: 2 routers + 2 switches + 2 DNS servers + 14 PCs (7 per department) = 20 devices.

3.1 Data Science Department — IP Addressing

| Device | IP Address | Subnet Mask | Notes |
| --- | --- | --- | --- |
| Router0 (Fa0/0) | 192.168.10.1 | 255.255.255.0 | Gateway for Network A |
| PC0 – PC6 | 192.168.10.2 – .8 | 255.255.255.0 | 7 workstations |
| Server0 (DNS) | 192.168.10.10 | 255.255.255.0 | DNS: server.datasci.local |
| Router0 (Se0/0/0) | 10.10.10.1 | 255.255.255.252 | Serial link to Router1 |

3.2 Computer Science Department — IP Addressing

| Device | IP Address | Subnet Mask | Notes |
| --- | --- | --- | --- |
| Router1 (Fa0/1) | 192.168.20.1 | 255.255.255.0 | Gateway for Network B |
| PC7 – PC13 | 192.168.20.2 – .8 | 255.255.255.0 | 7 workstations |
| Server1 (DNS) | 192.168.20.9 | 255.255.255.0 | DNS: server.comp-sci.local |
| Router1 (Se0/0/0) | 10.10.10.2 | 255.255.255.252 | Serial link to Router0 |

4. Configuration Summary

4.1 Routing

Static routes were configured on both routers so that each network can reach the other through the serial link:

- Router0: route to 192.168.20.0/24 via 10.10.10.2

- Router1: route to 192.168.10.0/24 via 10.10.10.1

4.2 DNS Configuration

Each department's server runs a DNS service with a records mapping every device name in its network to its IP address, including the server's own domain name (server.datasci.local and server.comp-sci.local). This allows devices to reach one another using names instead of raw IP addresses within the same network.

5. Testing & Results

Two categories of connectivity tests were performed to validate the design:

5.1 Ping Within the Same Network (by name)

Example: from PC9, running "ping Pc10" resolved the name via DNS to 192.168.20.3 and returned a successful reply (0% loss, TTL=128), confirming that both the DNS service and local connectivity function correctly.

5.2 Ping Between Different Networks (by IP address)

Example: from PC0 (Data Science, 192.168.10.x) to 192.168.20.2 (Computer Science) returned a successful reply with TTL=126 — one less than the TTL=128 seen for local traffic, confirming the packet correctly traversed the router hop between the two departmental networks.

All required ping tests completed successfully with 0% packet loss, verifying that both routing and DNS configurations work as intended across the full topology.

6. Conclusion

This project successfully demonstrates the design and configuration of two independent departmental networks connected through static routing, with functioning DNS resolution in each network. The topology reflects real-world network segmentation practices — separating departments into distinct subnets while maintaining full inter-department connectivity through a dedicated router link. All connectivity requirements (intra-network ping by name, inter-network ping by IP) were verified and passed successfully.

7. Deliverables

- Cisco Packet Tracer file (.pkt)

- This project report (.docx)

- Project presentation (.pptx)